# Accounting Area Check

Data Preprocessing and DTM Generation

Seamus Murphy

2021-12-05

On this page

Objective & Scope	1

Environment Setup	1

Data Inputs:	1

Methods	1

1. Loading Accounting Area	1

2. Geometric Validity Assessment	1

3. Total Accounting Area Calculation	1

4. Feature-Level Area Analysis	1

5. Topology and Overlap Detection	1

6. Boundary Vertex Density Analysis	1

7. Small Polygon Detection	1

8. Administrative Boundary Correspondence Check	1

9. Export Geometric Issues	1

10. Visualization	1

Findings	1

Key Findings	1

Key Observations	1

1. Total Area	1

2. Geometric Quality	1

3. Boundary Integrity	1

4. Recommendations	1

Next Steps	1

Session Information	1

## Objective & Scope

This document serves as a reference to methods applied in the area check for the State of Charagua, Bolivia submission to the Architecture for REDD+ Transactions (ART) TREES Program.

This audit verifies compliance with ART TREES Version 2.0 requirements for accounting area boundaries, particularly:

- Section 3.1.1 Subnational Accounting Requirements:

  - Minimum forest area of 2.5 million hectares at the beginning of the TREES Crediting Period

  - Boundaries correspond with entire administrative jurisdiction(s) or recognized Indigenous territories

  - Subnational crediting period ending December 31, 2030

  - National government authorization for subnational participant

The following tests evaluate the submitted accounting area shapefiles against these requirements and assess geometric validity, area calculations, and potential boundary issues.

### Environment Setup

easypackages::packages(
  "bslib", 
  "cols4all", 
  "dplyr",
  "exactextractr","elevatr", 
  "forestdata",
  "geos", "ggplot2", "ggstats", "ggspatial", "ggmap", "giscoR", 
  "hdf5r", "httr", "httr2", "htmltools",
  "knitr", "kableExtra",
  "leaflet.providers", "leafem", "leaflet", "leafgl", "libgeos","lwgeom", 
  "mapedit", "mapview", "maptiles", 
  "ncdf4",
  "openxlsx", 
  "rasterVis", "raster", "Rcpp", "RcppArmadillo", "RColorBrewer", "rts", 
  "s2", "sf", "scales", "sits","spdep", "stars", "stringr",
  "terra", "tidyverse", "tidyterra","tmap", "tmaptools", "terrainr", 
  "units",
  prompt = F)

#mapviewOptions(fgb = FALSE)
sf::sf_use_s2(use_s2 = FALSE)

### Data Inputs:

# Define file paths (update these paths as needed)
accounting_area_file <- "path/to/Bolivia_Charagua_AccountingArea.shp"
forest_area_file <- "path/to/Bolivia_Charagua_ForestArea.shp"  # If separate
admin_boundary_file <- "path/to/Bolivia_Admin_Boundaries.shp"  # Reference boundaries

# Display input information
cat("Accounting Area Shapefile:", accounting_area_file, "\n")
cat("Forest Area Shapefile:", forest_area_file, "\n")
cat("Administrative Boundary Reference:", admin_boundary_file, "\n")

## Methods

### 1. Loading Accounting Area

# Load accounting area shapefile
accounting_area <- st_read(accounting_area_file, quiet = TRUE)

# Display basic information
cat("Coordinate Reference System:\n")
print(st_crs(accounting_area))

cat("\nNumber of features:", nrow(accounting_area), "\n")
cat("Geometry type:", unique(st_geometry_type(accounting_area)), "\n")

# Display attribute table structure
cat("\nAttribute columns:\n")
str(accounting_area)

### 2. Geometric Validity Assessment

# Check for geometric validity
validity_check <- st_is_valid(accounting_area, reason = TRUE)

validity_summary <- data.frame(
  Feature_ID = 1:nrow(accounting_area),
  Is_Valid = st_is_valid(accounting_area),
  Validity_Reason = validity_check
)

kable(validity_summary, 
      caption = "Geometric Validity Check Results",
      col.names = c("Feature ID", "Valid", "Issue Description")) %>%
  kable_styling(bootstrap_options = c("striped", "hover", "condensed")) %>%
  row_spec(which(!validity_summary$Is_Valid), bold = TRUE, color = "white", background = "#d9534f")

# Fix invalid geometries if needed
if(any(!st_is_valid(accounting_area))) {
  cat("\nAttempting to fix invalid geometries using st_make_valid()...\n")
  accounting_area <- st_make_valid(accounting_area)
  
  cat("Post-fix validity check:\n")
  print(table(st_is_valid(accounting_area)))
}

### 3. Total Accounting Area Calculation

# Ensure proper projected CRS for area calculation
# Using an appropriate projection for Bolivia (e.g., UTM Zone 20S or custom)
bolivia_crs <- st_crs("+proj=utm +zone=20 +south +datum=WGS84 +units=m +no_defs")

accounting_area_proj <- st_transform(accounting_area, bolivia_crs)

# Calculate areas
accounting_area_proj$area_m2 <- st_area(accounting_area_proj)
accounting_area_proj$area_ha <- set_units(accounting_area_proj$area_m2, ha)

# Total area calculation
total_area_ha <- sum(accounting_area_proj$area_ha)
total_area_million_ha <- as.numeric(total_area_ha) / 1e6

# Display results
area_summary <- data.frame(
  Metric = c("Total Area (hectares)", 
             "Total Area (million hectares)",
             "Meets 2.5M ha threshold?"),
  Value = c(
    format(as.numeric(total_area_ha), big.mark = ",", scientific = FALSE),
    format(total_area_million_ha, digits = 3),
    ifelse(total_area_million_ha >= 2.5, "✓ YES", "✗ NO")
  )
)

kable(area_summary,
      caption = "Total Accounting Area - ART TREES Compliance Check") %>%
  kable_styling(bootstrap_options = c("striped", "hover")) %>%
  row_spec(3, bold = TRUE, 
           color = ifelse(total_area_million_ha >= 2.5, "white", "white"),
           background = ifelse(total_area_million_ha >= 2.5, "#5cb85c", "#d9534f"))

### 4. Feature-Level Area Analysis

# Individual feature areas
feature_areas <- accounting_area_proj %>%
  st_drop_geometry() %>%
  select(matches("NAME|ID|TYPE"), area_ha) %>%
  arrange(desc(area_ha))

kable(feature_areas,
      caption = "Individual Feature Areas",
      col.names = c(names(feature_areas)[1:(ncol(feature_areas)-1)], "Area (ha)"),
      digits = 2) %>%
  kable_styling(bootstrap_options = c("striped", "hover", "condensed"), 
                font_size = 12) %>%
  scroll_box(height = "400px")

### 5. Topology and Overlap Detection

# Check for overlaps between polygons
if(nrow(accounting_area_proj) > 1) {
  
  # Test for overlaps
  overlap_matrix <- st_overlaps(accounting_area_proj, sparse = FALSE)
  
  # Find overlapping features (excluding self-overlaps)
  diag(overlap_matrix) <- FALSE
  overlap_exists <- any(overlap_matrix)
  
  if(overlap_exists) {
    overlap_pairs <- which(overlap_matrix, arr.ind = TRUE)
    overlap_df <- data.frame(
      Feature_1 = overlap_pairs[,1],
      Feature_2 = overlap_pairs[,2]
    )
    
    cat("WARNING: Overlapping features detected!\n\n")
    kable(overlap_df, caption = "Overlapping Feature Pairs") %>%
      kable_styling(bootstrap_options = c("striped", "hover")) %>%
      row_spec(0:(nrow(overlap_df)), background = "#f0ad4e")
  } else {
    cat("✓ No overlaps detected between features\n")
  }
  
  # Check for gaps
  union_geom <- st_union(accounting_area_proj)
  dissolved_geom <- st_make_valid(st_buffer(union_geom, dist = 0))
  
  # Area comparison to detect potential gaps
  union_area <- st_area(union_geom)
  dissolved_area <- st_area(dissolved_geom)
  area_diff_pct <- abs(as.numeric(union_area - dissolved_area) / as.numeric(union_area) * 100)
  
  cat("\nGap Analysis:\n")
  cat("Union area:", format(as.numeric(union_area), scientific = FALSE), "m²\n")
  cat("Dissolved area:", format(as.numeric(dissolved_area), scientific = FALSE), "m²\n")
  cat("Difference:", format(area_diff_pct, digits = 4), "%\n")
  
  if(area_diff_pct > 0.01) {
    cat("⚠ Potential gaps or slivers detected (difference >0.01%)\n")
  } else {
    cat("✓ No significant gaps detected\n")
  }
} else {
  cat("Single feature - no overlap/gap analysis needed\n")
}

### 6. Boundary Vertex Density Analysis

# Calculate vertices per feature
vertex_counts <- sapply(st_geometry(accounting_area_proj), function(g) {
  coords <- st_coordinates(g)
  nrow(coords)
})

vertex_summary <- data.frame(
  Feature = 1:length(vertex_counts),
  Vertex_Count = vertex_counts,
  Perimeter_m = as.numeric(st_length(st_cast(accounting_area_proj, "MULTILINESTRING"))),
  Vertices_per_km = vertex_counts / (as.numeric(st_length(st_cast(accounting_area_proj, "MULTILINESTRING"))) / 1000)
)

kable(vertex_summary,
      caption = "Boundary Complexity Analysis",
      digits = 1) %>%
  kable_styling(bootstrap_options = c("striped", "hover", "condensed"))

# Summary statistics
cat("\nVertex Density Statistics:\n")
cat("Mean vertices per km:", mean(vertex_summary$Vertices_per_km), "\n")
cat("Median vertices per km:", median(vertex_summary$Vertices_per_km), "\n")
cat("Max vertices per km:", max(vertex_summary$Vertices_per_km), "\n")

### 7. Small Polygon Detection

# Identify potentially problematic small polygons
min_area_threshold <- 100  # hectares

small_polys <- accounting_area_proj %>%
  filter(as.numeric(area_ha) < min_area_threshold) %>%
  st_drop_geometry() %>%
  select(matches("NAME|ID|TYPE"), area_ha) %>%
  arrange(area_ha)

if(nrow(small_polys) > 0) {
  cat("WARNING:", nrow(small_polys), "features found below", min_area_threshold, "ha threshold\n\n")
  
  kable(small_polys,
        caption = paste0("Small Polygons (<", min_area_threshold, " ha)"),
        digits = 2) %>%
    kable_styling(bootstrap_options = c("striped", "hover", "condensed")) %>%
    row_spec(0:nrow(small_polys), background = "#f0ad4e")
} else {
  cat("✓ No polygons smaller than", min_area_threshold, "ha detected\n")
}

### 8. Administrative Boundary Correspondence Check

# This section requires reference administrative boundaries
# Load official administrative boundaries
admin_boundaries <- st_read(admin_boundary_file, quiet = TRUE)
admin_boundaries_proj <- st_transform(admin_boundaries, bolivia_crs)

# Filter to relevant jurisdiction (update filter as needed)
charagua_admin <- admin_boundaries_proj %>%
  filter(grepl("Charagua", NAME, ignore.case = TRUE))

# Calculate overlap percentage
overlap_area <- st_intersection(accounting_area_proj, charagua_admin)
overlap_pct <- as.numeric(st_area(overlap_area)) / as.numeric(st_area(accounting_area_proj)) * 100

correspondence_check <- data.frame(
  Check = "Boundary Correspondence",
  Result = ifelse(overlap_pct > 99.9, "✓ PASS", "✗ FAIL"),
  Overlap_Percentage = paste0(format(overlap_pct, digits = 4), "%")
)

kable(correspondence_check,
      caption = "Administrative Boundary Correspondence") %>%
  kable_styling(bootstrap_options = c("striped", "hover")) %>%
  row_spec(1, bold = TRUE,
           color = ifelse(overlap_pct > 99.9, "white", "white"),
           background = ifelse(overlap_pct > 99.9, "#5cb85c", "#d9534f"))

### 9. Export Geometric Issues

# Compile all geometric issues for further investigation
issues_list <- list()

# Invalid geometries
if(any(!validity_summary$Is_Valid)) {
  invalid_geoms <- accounting_area[!validity_summary$Is_Valid, ]
  st_write(invalid_geoms, 
           "/mnt/user-data/outputs/Bolivia_Invalid_Geometries.shp",
           delete_dsn = TRUE)
  issues_list$invalid_geometries <- nrow(invalid_geoms)
}

# Small polygons
if(nrow(small_polys) > 0) {
  small_poly_geoms <- accounting_area_proj %>%
    filter(as.numeric(area_ha) < min_area_threshold)
  st_write(st_transform(small_poly_geoms, 4326),
           "/mnt/user-data/outputs/Bolivia_Small_Polygons.shp",
           delete_dsn = TRUE)
  issues_list$small_polygons <- nrow(small_poly_geoms)
}

# Summary of exported files
if(length(issues_list) > 0) {
  cat("Geometric issues exported to shapefiles:\n")
  for(issue in names(issues_list)) {
    cat(" -", issue, ":", issues_list[[issue]], "features\n")
  }
} else {
  cat("✓ No geometric issues requiring export\n")
}

### 10. Visualization

# Create map of accounting area
ggplot() +
  geom_sf(data = accounting_area, 
          aes(fill = as.numeric(st_area(accounting_area))/10000),
          color = "darkblue", size = 0.5) +
  scale_fill_viridis_c(
    name = "Area (ha)",
    labels = scales::comma,
    trans = "log10"
  ) +
  labs(
    title = "ART TREES Accounting Area - State of Charagua, Bolivia",
    subtitle = paste0("Total Area: ", 
                     format(as.numeric(total_area_ha), big.mark = ","), 
                     " ha (", 
                     format(total_area_million_ha, digits = 3), 
                     " million ha)"),
    caption = paste0("Source: TREES Registration Document\n",
                    "Projection: ", st_crs(accounting_area)$input)
  ) +
  theme_minimal() +
  theme(
    plot.title = element_text(size = 16, face = "bold"),
    plot.subtitle = element_text(size = 12),
    legend.position = "right",
    panel.grid.major = element_line(color = "gray90"),
    panel.background = element_rect(fill = "aliceblue")
  ) +
  coord_sf()

# Create additional detail map if multiple features
if(nrow(accounting_area) > 1) {
  ggplot() +
    geom_sf(data = accounting_area, 
            aes(fill = factor(1:nrow(accounting_area))),
            color = "black", size = 0.3) +
    scale_fill_viridis_d(name = "Feature ID") +
    labs(
      title = "Accounting Area Features",
      subtitle = paste0(nrow(accounting_area), " distinct features")
    ) +
    theme_minimal() +
    theme(
      plot.title = element_text(size = 14, face = "bold"),
      legend.position = "right"
    ) +
    coord_sf()
}

# Findings

## Key Findings

# Compile findings
findings <- data.frame(
  Requirement = c(
    "Minimum 2.5 million hectares",
    "Geometric validity",
    "No overlaps between features",
    "No significant gaps",
    "Administrative boundary correspondence"
  ),
  Status = c(
    ifelse(total_area_million_ha >= 2.5, "✓ PASS", "✗ FAIL"),
    ifelse(all(st_is_valid(accounting_area)), "✓ PASS", "⚠ WARNING"),
    ifelse(!overlap_exists || nrow(accounting_area) == 1, "✓ PASS", "✗ FAIL"),
    ifelse(area_diff_pct <= 0.01 || nrow(accounting_area) == 1, "✓ PASS", "⚠ WARNING"),
    "PENDING"  # Update based on actual check
  ),
  Value = c(
    paste0(format(total_area_million_ha, digits = 3), " million ha"),
    paste0(sum(st_is_valid(accounting_area)), "/", nrow(accounting_area), " valid"),
    ifelse(!overlap_exists || nrow(accounting_area) == 1, "None detected", 
           paste(nrow(overlap_df), "overlaps")),
    ifelse(area_diff_pct <= 0.01, "<0.01%", paste0(format(area_diff_pct, digits = 2), "%")),
    "Requires reference data"
  )
)

kable(findings,
      caption = "ART TREES Area Check - Compliance Summary") %>%
  kable_styling(bootstrap_options = c("striped", "hover", "condensed")) %>%
  column_spec(2, bold = TRUE) %>%
  row_spec(which(findings$Status == "✓ PASS"), background = "#d4edda") %>%
  row_spec(which(findings$Status == "⚠ WARNING"), background = "#fff3cd") %>%
  row_spec(which(findings$Status == "✗ FAIL"), background = "#f8d7da") %>%
  row_spec(which(findings$Status == "PENDING"), background = "#e7f3ff")

## Key Observations

### 1. Total Area

The submitted accounting area shapefile contains

-   r format(as.numeric(total_area_ha), big.mark = ",")
    hectares
-   (r format(total_area_million_ha, digits = 3) million
    hectares).

Compliance Assessment:

- r if(total_area_million_ha >= 2.5) { paste0(“✓ The accounting area MEETS the ART TREES requirement of at least 2.5 million hectares of forest area.”) } else { paste0(“✗ The accounting area DOES NOT MEET the ART TREES requirement of at least 2.5 million hectares. The submitted area is”, format((2.5 - total_area_million_ha) * 1e6, big.mark = “,”), ” hectares short of the minimum threshold.”) }

### 2. Geometric Quality

-   Valid geometries: `r sum(st_is_valid(accounting_area))`
    / `r nrow(accounting_area)`
-   Features with issues:
    `r sum(!st_is_valid(accounting_area))`

`r if(any(!st_is_valid(accounting_area))) { paste0("⚠ Geometric validity issues were detected and require correction. Invalid geometries have been exported for review.") } else { "✓ All features have valid geometry." }`

### 3. Boundary Integrity

- Overlapping features:

`r if(!overlap_exists || nrow(accounting_area) == 1) { "None detected" } else { paste(nrow(overlap_df), "overlap(s) detected") }`
-   Gap analysis:
    `r if(area_diff_pct <= 0.01 || nrow(accounting_area) == 1) { "No significant gaps" } else { paste0("Potential gaps detected (", format(area_diff_pct, digits = 2), "% area difference)") }`

### 4. Recommendations

Based on this preliminary area check:

- If area threshold not met: The Participant must revise the accounting area boundaries to include sufficient forest area to meet the 2.5 million hectare minimum requirement.

- Geometric issues: Any invalid geometries, overlaps, or gaps must be resolved before final registration.

- Administrative correspondence: Verify that boundaries correspond exactly with the official administrative boundaries of the State of Charagua (or relevant Indigenous territories).

- Authorization: Confirm that the national government authorization letter is included in the registration package and explicitly references the accounting area boundaries.

- Crediting period: Verify that the proposed crediting period end date does not extend beyond December 31,

## Next Steps

- Resolve any geometric validity issues

- Verify administrative boundary correspondence with official sources

- Cross-reference with forest cover data to confirm forest area calculation

- Review national government authorization letter

- Confirm crediting period dates

- Conduct additional stratification and forest definition checks

## Session Information

sessionInfo()

Auditor Notes:

This area check provides a preliminary assessment of geometric and area requirements. Additional verification steps may be required, including:

- Forest cover classification validation

- Historical forest area trends

- Deforestation/degradation activity data alignment

- Carbon stock stratification assessment

- Comparison with national forest inventory data